BLANK GENERATOR

This function is developed specially for meshing blank.

User can create blank mesh from the existing surface and lines. Select BLANK GENERATOR, the SELECT OPTION dialog window is displayed as Figure 9.5.1.

Figure 9.5.1 Select Option for Blank Generator

- Select BOUNDARY LINE, a SELECT LINE window is displayed or Select SURFACE, the SELECT SURFACE window is displayed, you can only select one surface by CURSOR

- After selecting the blank boundary line, the CONCERN TOOL RADII window appears. Input proper value (refer to training).

- A DYNAFORM QUESTION window prompt and ask you whether you accept the mesh.

YES –	Accept the mesh
NO – 	Cancel the operation
REMESH -	CONCERN TOOL RADII window will be displayed again and allow user to               adjust radius.